Comprehensive Guide to Unboxing, Setting Up, and Maintaining Your New Hot Tub

- Safety First:

  - Upon receiving your brand-new hot tub and your electrician has powered the unit, as the first step, prioritize safety by completely shutting down the hot tub. If it is a 220V unit, switch it off from the breaker; if it is 110V, unplug it.

- Filling the Tub:

  - REMOVE all filters. Start filling the hot tub with water. During this process, it is crucial to ensure that the water flows down the hole where the filter usually sits. This allows water to enter through the plumbing and emerge from the inside of the jets. Do not activate the jets at this point to prevent potential airlocks.

- .;.;

  - Keep a close eye on the water level. Once it reaches approximately 1 inch below the headrests, stop filling the hot tub.

- Filter Replacement:

  - Carefully reinsert the filter after the water filled, making sure it is properly threaded and securely fitted into the unit. The filter plays a critical role in maintaining water quality.

- Electrical Activation:

  - With the hot tub filled to 1” under the headrests, proceed to turn it on. For 220V models, restore power from the breaker; for 110V units, plug it in. This will power the unit and it will enter an Initial Startup Phase.

- Initial Startup Phase:

  - When turning on the hot tub, it will go through an initial startup phase, which typically takes about 15 minutes. It is of utmost importance that during this phase, you refrain from touching any buttons, adjusting the temperature, or activating the pumps. Allow the hot tub to complete its startup independently.

Using the @ease Mineral Sanitizer System:

- Understanding the Kit:

  - Your @ease starter kit includes two halves of a ball: the FROG @ease Mineral Cartridge (top half in blue) and the FROG @ease SmartChlor® Cartridge (bottom half in silver). The package also contains specific test strips tailored to this chemical system.

**The @ease Mineral Sanitizer system operates differently from traditional chlorine-based systems. Be aware that if you change chemicals or have your water tested elsewhere, the results may not be accurate, as this system relies on minerals for sanitization.

- Activating the (Blue Mineral Portion):

  - The blue mineral portion is effective for approximately 4 months and is not separately packaged. To activate it during the initial fill and each time you drain and refill the hot tub or replace the blue portion, use the Jump start. Distribute the Jump start evenly throughout the hot tub, placing it in each corner or in the outer area if the hot tub is round.

- SmartChlor (Silver portion):

- The silver portion is separately packaged in the kit. The silver portion lasts between 3-4 weeks. There is a dial on bottom portion of the silver portion, the lower the dial setting, the closer it will last to 4 weeks. The higher the setting the closer it will last to 3 weeks. We recommend putting the silver on dial setting "2" and if you feel like you need more sanitization in the water, only then will you increase it.

- Connecting the Cartridges:

  - After adding the Jump start to activate the minerals (blue portion), connect the silver and blue portions together. Look at the bottom of each component to see how they clip together to form a complete ball, with both silver and blue portions connected.

- Placing the Complete System:

- Gently place the connected, complete @ease Mineral Sanitizer system into the hot tub water, and allow it to float.

**Bullfrog Spas @ease mineral sanitizer system goes into a jet pack inside of the hot tub and DO NOT float, however they work and operate the same.

Post-Startup Steps:

- Temperature Adjustment:

  - After the 15-minute initial startup phase, check if your hot tub displays the temperature. If it only shows the temperature, it is ready for use. At this point, you can adjust the spa's "set temperature." For example, in Florida, a range of 92-98 degrees is energy-efficient for hot tub operation. The hot tub will autonomously regulate the temperature and filter the water, eliminating the need for manual button presses.

13.      Filter Settings:

  - It is crucial to avoid adjusting the filter duration or settings, as they come pre-programmed. The hot tub will automatically filter every 12 hours and initiate additional filtration when it detects a slowdown in water pressure, indicating the need for cleaning.

- Additional Features:

  - You can activate the pumps and lights when you are ready to use the hot tub. If your hot tub has a timer, set the time as needed. The hot tub will handle temperature regulation and water filtration independently.

Maintenance Tips:

- Draining and Refilling:

  - To maintain water quality, plan to drain and refill the hot tub approximately every 4 months or around 3 times a year. This schedule aligns well with replacing the blue portion of the @ease Mineral Sanitizer, which lasts for 4 months.

- Acrylic Cleaning:

When you are draining and refilling the hot tub, take the opportunity to clean the acrylic surface without water. Avoid using soapy substances; instead, use plain water with a terry or microfiber cloth. Soapy residues can accumulate in the filter and cause excessive foaming.

- Efficient Draining Method:

  - Do not use the hot tub's drain plug, as it drains water slowly due to gravity. For a more efficient method, consider reverse siphoning with your garden hose or invest in a submersible pump (typically costing between $30-100). To reverse siphon, attach your garden hose as if you were filling the hot tub, place the hose's end directly in the bottom middle of the hot tub, and hold it down with your cover. Once the water starts flowing, go to the spigot and unscrew it. This will create a reverse flow of water, allowing you to drain it quickly into a drain or onto the grass.

- Maintaining the Mineral Sanitizer:

  - Keep in mind that the @ease mineral sanitizer cannot be allowed to dry out, as this will deactivate the minerals. If the floating ball is bothering you during hot tub use, place it in a bucket or bowl of hot tub water. Always return the ball to the hot tub water when finished using the hot tub.

When refilling the hot tub, remember to follow the same steps as you did during the initial fill, including adding Jump start to activate the new blue mineral portion. By adhering to these guidelines, you will ensure the proper setup and long-term enjoyment of your hot tub, while effectively utilizing the @ease Mineral Sanitizer system. So, sit back, relax, and relish your hot tub experience to the fullest!

Do not worry about registering your hot tub, we will do it for you!

Congratulations on your new hot tub

This Photo by Unknown Author is licensed under CC BY-SA

-Team Unique